alu-4bit/
 ├── alu.v
 ├── alu_tb.v
 ├── README.md

alu.v

module alu (
    input [3:0] A,
    input [3:0] B,
    input [2:0] sel,
    output reg [3:0] result,
    output reg carry
);

always @(*) begin
    case (sel)
        3'b000: {carry, result} = A + B;   // Addition
        3'b001: {carry, result} = A - B;   // Subtraction
        3'b010: result = A & B;            // AND
        3'b011: result = A | B;            // OR
        3'b100: result = A ^ B;            // XOR
        3'b101: result = ~A;               // NOT
        3'b110: result = A << 1;           // Shift Left
        3'b111: result = A >> 1;           // Shift Right
        default: result = 4'b0000;
    endcase
end

endmodule

alu_tb.v

`timescale 1ns/1ps

module alu_tb;

reg [3:0] A, B;
reg [2:0] sel;
wire [3:0] result;
wire carry;

alu uut (
    .A(A),
    .B(B),
    .sel(sel),
    .result(result),
    .carry(carry)
);

initial begin
    $monitor("A=%b B=%b sel=%b result=%b carry=%b", A, B, sel, result, carry);

    A = 4'b0101; B = 4'b0011;

    sel = 3'b000; #10; // Add
    sel = 3'b001; #10; // Sub
    sel = 3'b010; #10; // AND
    sel = 3'b011; #10; // OR
    sel = 3'b100; #10; // XOR
    sel = 3'b101; #10; // NOT
    sel = 3'b110; #10; // Shift Left
    sel = 3'b111; #10; // Shift Right

    $finish;
end

endmodule

README.md

# 4-bit ALU in Verilog

## Description
This project implements a 4-bit Arithmetic Logic Unit (ALU) using Verilog HDL.

## Operations Supported
- Addition
- Subtraction
- AND, OR, XOR
- NOT
- Shift Left, Shift Right

## Tools Used
- Verilog HDL
- ModelSim / Vivado (for simulation)

## Learning Outcomes
- Combinational logic design
- Case statement usage
- Testbench development

Project 2: Traffic Light Controller

traffic-light-fsm/
 ├── traffic.v
 ├── traffic_tb.v
 ├── README.md

traffic.v

module traffic_light (
    input clk,
    input reset,
    output reg [1:0] state
);

// States
parameter RED = 2'b00, GREEN = 2'b01, YELLOW = 2'b10;

always @(posedge clk or posedge reset) begin
    if (reset)
        state <= RED;
    else begin
        case (state)
            RED: state <= GREEN;
            GREEN: state <= YELLOW;
            YELLOW: state <= RED;
            default: state <= RED;
        endcase
    end
end

endmodule

traffic_tb.v

`timescale 1ns/1ps

module traffic_tb;

reg clk, reset;
wire [1:0] state;

traffic_light uut (
    .clk(clk),
    .reset(reset),
    .state(state)
);

always #5 clk = ~clk;

initial begin
    clk = 0;
    reset = 1;
    #10 reset = 0;

    #100 $finish;
end

endmodule

README Highlights

# Traffic Light Controller using FSM

## Concepts Used
- Finite State Machine (FSM)
- Sequential logic
- Clock-driven design

## States
- RED → GREEN → YELLOW → RED

Project 3: 4-bit Synchronous Counter

counter.v

module counter (
    input clk,
    input reset,
    output reg [3:0] count
);

always @(posedge clk or posedge reset) begin
    if (reset)
        count <= 4'b0000;
    else
        count <= count + 1;
end

endmodule